# 1. Objective

To design a proportional controller for a given digital transfer function using discrete root-locus analysis and to determine the controller gain required to meet the closed-loop specifications %OS ≤ 5 % and ts ≤ 9 s. The effect of sampling time is studied by repeating the design for T = 0.1 s to 1 s in steps of 0.1 s.

# 2. Theory

With a proportional controller C(z) = Kp and unity negative feedback, the open-loop function is L(z) = KpG(z) and the closed-loop poles are the roots of 1 + KpG(z) = 0; as Kp varies they trace the discrete root locus. The s- and z-planes are related by z = esT: for s = σ + jω, |z| = eσT and ∠z = ωT, so pole locations in the z-plane depend on T, and the stable region is the interior of the unit circle. zgrid shows constant-ζ and constant-ωn curves in this plane.

The 5 % overshoot limit corresponds to the minimum damping ratio

and, for the digital loop, the damping of a z-plane pole is measured by the effective damping ratio (independent of T, because s = ln z / T only rescales |s|):

# 3. Given Digital Transfer Function

At T = 1 s the given plant is G(z) = 0.32(z + 0.62)/(z² − 1.15z + 0.15), defined in MATLAB as

z = tf('z',1); Gz_1 = 0.32*(z + 0.62)/((z - 1)*(z - 0.15));

# 4. Continuous-Time Equivalent of the Plant

The digital plant is converted to a continuous equivalent with the zero-order-hold inverse (Gs = d2c(Gz_1,’zoh’)). The MATLAB result — reproduced here exactly in closed form (§17) — is

The poles are s = 0 and s = −1.897; the zero is at s = +31.18 (non-minimum phase, far above the sampling frequency for every T used, so it affects the discrete zero but not the dominant dynamics). The required damping ratio is ζreq = 0.6901.

Analog gain window. Closing the loop around the continuous plant and applying the same specifications gives, by an exact search of the continuous step response, 0.582 ≤ Kp ≤ 1.536. ◆ Your report quotes ≈ 0.678 and ≈ 1.845. Those two numbers do not satisfy your own specification for this plant: at Kp = 0.678 the continuous ts is already 7.33 s (the 9 s boundary is at 0.582), and at Kp = 1.845 the overshoot is 8.05 % (the 5 % boundary is at 1.536). The corrected values are also the ones towards which the digital limits converge as T → 0 (the ζ-crossing gain is 1.522 at T = 0.01 s), so they are the right reference for §11 and for the discussion of “analog vs digital” gains. Figure 1 shows the continuous step responses at the two corrected limits.

Figure 1. Left: loop gain at each T’s Kp,max against the analog G(jω) (Kp = 1): the sampled loop follows the analog one up to ≈ π/T and then rolls off. Right: continuous-plant step responses at the corrected analog limits.

# 5. Discretisation for Different Sampling Times

For T = 0.1, 0.2, …, 1.0 s the continuous plant is discretised by ZOH, Gd = c2d(Gs,T,’zoh’). Writing Gs = c0/s + c1/(s+a) (a = 1.8971, c0 = 0.6099, c1 = −0.6470), every plant has the form (b1z + b0)/[(z−1)(z−p)] with

| T (s) | b1 | b0 | z coefficient: −(1+p) | p = e−aT | zero −b0/b1 |
| --- | --- | --- | --- | --- | --- |
| 0.1 | 0.002056 | 0.008483 | -1.82720 | 0.82720 | -4.126 |
| 0.2 | 0.014296 | 0.024218 | -1.68426 | 0.68426 | -1.694 |
| 0.3 | 0.034959 | 0.044445 | -1.56601 | 0.56601 | -1.271 |
| 0.4 | 0.062591 | 0.067142 | -1.46821 | 0.46821 | -1.073 |
| 0.5 | 0.095987 | 0.090851 | -1.38730 | 0.38730 | -0.947 |
| 0.6 | 0.134150 | 0.114545 | -1.32037 | 0.32037 | -0.854 |
| 0.7 | 0.176258 | 0.137521 | -1.26501 | 0.26501 | -0.780 |
| 0.8 | 0.221629 | 0.159320 | -1.21922 | 0.21922 | -0.719 |
| 0.9 | 0.269698 | 0.179662 | -1.18134 | 0.18134 | -0.666 |
| 1.0 | 0.320000 | 0.198400 | -1.15000 | 0.15000 | -0.620 |

Table 1. Sampling time versus discrete-time plant Gd(z) = (b1z + b0)/(z² − (1+p)z + p). At T = 1 s this returns the given 0.32z + 0.1984 over z² − 1.15z + 0.15 exactly.

The finite zero moves from −4.13 at T = 0.1 s (outside the unit circle) to −0.62 at T = 1 s; it crosses the unit circle at Tc = 0.453 s (§15).

# 6. Discrete Root-Locus Analysis

For each T the root locus of KpGd(z) is obtained by varying Kp; the unit circle is the stability boundary. Figure 2 shows all ten loci with the ζ = 0.6901 contour and the two gain boundaries of §7 marked (▲ Kp,min, ■ Kp,max). At every T the lower-gain pair of poles is real (▲, between the two open-loop poles), meets and leaves the real axis at breakaway, and the complex branch crosses the ζ = 0.6901 contour at Kp,max (■) on its way to the unit circle.

Figure 2. Discrete root loci for T = 0.1 … 1.0 s with the ζ = 0.6901 contour (dotted) and the unit circle (dashed).

# 7. Proportional Controller Design and Gain Results

For each T the gain is swept; for every Kp the closed-loop transfer function Tcl(z) = KpGd(z)/(1 + KpGd(z)) is built and stepinfo supplies the 2 % settling time and the peak overshoot. A gain is acceptable only if %OS ≤ 5 and ts ≤ 9 s, so for every T

(the settling-time limit fixes the lower edge and the overshoot limit the upper edge). Your sweep, with the detailed values at both edges, is reproduced in Table 2. Because a stepinfo sweep has a finite gain step (0.01 in your table) and settling time is quantised in multiples of T, the edges were also located exactly by a 0.001 scan followed by bisection (Table 3). The two agree within 0.01 everywhere except the lower edge at T = 0.3 s (0.016), where the quantised ts makes the edge jump (§10).

| T (s) | Kp,min | ts (s) | Mp (%) | Kp,max | ts (s) | Mp (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.1 | 0.571 | 9 | 0 | 1.401 | 4.8 | 4.961 |
| 0.2 | 0.561 | 9 | 0 | 1.291 | 5 | 4.996 |
| 0.3 | 0.561 | 8.7 | 0 | 1.191 | 5.4 | 4.902 |
| 0.4 | 0.551 | 8.8 | 0 | 1.111 | 5.6 | 4.981 |
| 0.5 | 0.531 | 9 | 0 | 1.041 | 6 | 4.997 |
| 0.6 | 0.511 | 9 | 0 | 0.971 | 6 | 4.857 |
| 0.7 | 0.531 | 8.4 | 0 | 0.921 | 6.3 | 4.843 |
| 0.8 | 0.501 | 8.8 | 0 | 0.861 | 7.2 | 4.766 |
| 0.9 | 0.481 | 9 | 0 | 0.831 | 7.2 | 4.966 |
| 1.0 | 0.471 | 9 | 0 | 0.781 | 7 | 4.884 |

Table 2. Your sweep results (lab report), reproduced unchanged.

| T (s) | Kp,min (yours) | Kp,min exact | ts | OS % | Kp,max (yours) | Kp,max exact | ts | OS % | ΔKp exact |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 | 0.571 | 0.5694 | 9.0 | 0.0 | 1.401 | 1.4043 | 4.8 | 5.000 | 0.835 |
| 0.2 | 0.561 | 0.5571 | 9.0 | 0.0 | 1.291 | 1.2913 | 5.0 | 5.000 | 0.734 |
| 0.3 | 0.561 | 0.5451 | 9.0 | 0.0 | 1.191 | 1.1984 | 5.4 | 5.000 | 0.653 |
| 0.4 | 0.551 | 0.5415 | 8.8 | 0.0 | 1.111 | 1.1121 | 5.6 | 5.000 | 0.571 |
| 0.5 | 0.531 | 0.5218 | 9.0 | 0.0 | 1.041 | 1.0411 | 6.0 | 5.000 | 0.519 |
| 0.6 | 0.511 | 0.5105 | 9.0 | 0.0 | 0.971 | 0.9766 | 6.0 | 5.000 | 0.466 |
| 0.7 | 0.531 | 0.5210 | 8.4 | 0.0 | 0.921 | 0.9259 | 6.3 | 5.000 | 0.405 |
| 0.8 | 0.501 | 0.4954 | 8.8 | 0.0 | 0.861 | 0.8709 | 7.2 | 5.000 | 0.375 |
| 0.9 | 0.481 | 0.4783 | 9.0 | 0.0 | 0.831 | 0.8319 | 7.2 | 5.000 | 0.354 |
| 1.0 | 0.471 | 0.4681 | 9.0 | 0.0 | 0.781 | 0.7846 | 7.0 | 5.000 | 0.316 |

Table 3. Exact gain window for your plant (csv_user_plant/table1_digital_design.csv). Your values are conservative by ≤ 0.01 at the upper edge; the exact upper edge reaches OS = 5.000 % and the exact lower edge ts = 9 s (8.8 s at T = 0.4 and 0.8 s, 8.4 s at 0.7 s because ts moves in steps of T).

Figure 3 shows the closed-loop step responses at both edges for T = 0.1, 0.5 and 1 s; the staircase is the sampled output. Figure 4 plots the window against T, with the exact edges, your tabulated values, the fitted laws of §11 and the continuous-plant limits.

Figure 3. Step responses at Kp,min and Kp,max (T = 0.1, 0.5, 1.0 s). Dotted: ±2 % band; dashed red: +5 %; dash-dot: 9 s.

Figure 4. Admissible gain window versus sampling time. Crosses are the values of your table; the dotted lines are the continuous-plant limits.

# 8. Analysis of the Gain Results

The specifications can be met at every sampling time. At T = 0.1 s the acceptable range is 0.571 ≤ Kp ≤ 1.401 (exact 0.569 ≤ Kp ≤ 1.404); at T = 1.0 s it is 0.471 ≤ Kp ≤ 0.781 (exact 0.468 ≤ Kp ≤ 0.785). The width ΔKp = Kp,max − Kp,min falls from 0.830 to 0.310 in your table (exact 0.835 → 0.316, a 62 % reduction), so the controller-gain margin is much smaller at the larger sampling time.

Settling time. At the minimum acceptable gains ts sits at the 9 s limit (8.4–9 s); at the maximum gains it stays below 9 s and increases with T, from 4.8 s at T = 0.1 s to 7.0 s at T = 1 s (linear fit ts ≈ 2.81T + 4.51, R² = 0.94, Figure 5, left): the fastest-allowed response gets slower as sampling gets coarser. Overshoot. The overshoot is 0 % at Kp,min for all T and 4.7–5.0 % at Kp,max (Figure 5, right); your tabulated Kp,max values give 4.77–4.996 % because the sweep stops at the last gain below 5 %.

Figure 5. Left: settling time at both gain edges. Right: overshoot at both edges (crosses: your tabulated overshoot at your Kp,max).

— Part II: Added Analysis for Your Plant —

# 9. Exact Gain Window and Stability Limits

Beyond the two specification edges, the root-locus equations of the plant give three more gains in closed form. The characteristic equation is z² + [Kpb1 − (1+p)]z + [p + Kpb0] = 0, so the breakaway gain (poles turn complex, ζ = 1) is the smaller root of the discriminant equation, the ζ = 0.6901 crossing follows from ζeff(z) = 0.6901 on the complex branch, and — because the product of the pole pair is the constant term — the exact stability limit is

| T (s) | Breakaway (ζ=1) | Kp,min / breakaway | ζ=0.69 crossing | crossing / Kp,max | true OS at crossing (%) | Kp,stab | Kp,stab/Kp,max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 | 0.7205 | 0.79 | 1.4039 | 1.000 | 4.99 | 20.371 | 14.51× |
| 0.2 | 0.6881 | 0.81 | 1.2915 | 1.000 | 5.00 | 13.038 | 10.10× |
| 0.3 | 0.6575 | 0.83 | 1.1951 | 0.997 | 4.96 | 9.765 | 8.15× |
| 0.4 | 0.6286 | 0.86 | 1.1118 | 1.000 | 4.99 | 7.920 | 7.12× |
| 0.5 | 0.6014 | 0.87 | 1.0392 | 0.998 | 4.95 | 6.744 | 6.48× |
| 0.6 | 0.5759 | 0.89 | 0.9755 | 0.999 | 4.97 | 5.933 | 6.08× |
| 0.7 | 0.5519 | 0.94 | 0.9193 | 0.993 | 4.79 | 5.345 | 5.77× |
| 0.8 | 0.5293 | 0.94 | 0.8693 | 0.998 | 4.96 | 4.901 | 5.63× |
| 0.9 | 0.5082 | 0.94 | 0.8246 | 0.991 | 4.72 | 4.557 | 5.48× |
| 1.0 | 0.4883 | 0.96 | 0.7845 | 1.000 | 5.00 | 4.284 | 5.46× |

Table 4. Root-locus boundaries for your plant, to be read with the window of Table 3 (Kp,min, Kp,max). csv_user_plant/table1_digital_design.csv.

Three results stand out. (i) The lower edge lies below the breakaway gain at every T (0.468 against 0.488 at T = 1 s; 0.569 against 0.721 at T = 0.1 s): the gain that just meets ts ≤ 9 s still has two real poles, which is why the overshoot there is exactly 0 % and why your “mainly governed by settling time” statement is precisely right. (ii) The ζ = 0.6901 crossing reproduces the exact 5 % overshoot gain to within 1 % at all ten T (true overshoot 4.72–5.00 %), even though the plant has a zero; the crossing is therefore a closed-form way to find Kp,max without any step sweep. (iii) The stability limit is 5.5× to 14.5× above Kp,max (4.284 against 0.785 at T = 1 s, 20.37 against 1.404 at T = 0.1 s), so the design is nowhere near instability, but the cliff is much closer at coarse sampling. At T = 1 s the closed-loop poles are 0.585 and 0.415 at Kp,min, a double pole at 0.497 at breakaway, 0.4495 ± 0.3219j at Kp,max, and −0.1105 ± 0.9939j (on the unit circle) at Kp,stab. A gain 10 % above Kp,max gives 6.7–7.7 % overshoot at every T, so the overshoot edge is a hard one in practice.

# 10. Settling Time at the Lower Gain Boundary

| T (s) | slow real pole | σ = −ln¦z¦/T (1/s) | τ = 1/σ (s) | ln 50 / σ (s) | simulated ts (s) |
| --- | --- | --- | --- | --- | --- |
| 0.1 | 0.953 | 0.486 | 2.06 | 8.05 | 9.0 |
| 0.2 | 0.907 | 0.488 | 2.05 | 8.02 | 9.0 |
| 0.3 | 0.863 | 0.491 | 2.04 | 7.97 | 9.0 |
| 0.4 | 0.816 | 0.509 | 1.97 | 7.69 | 8.8 |
| 0.5 | 0.780 | 0.498 | 2.01 | 7.86 | 9.0 |
| 0.6 | 0.740 | 0.502 | 1.99 | 7.79 | 9.0 |
| 0.7 | 0.673 | 0.566 | 1.77 | 6.91 | 8.4 |
| 0.8 | 0.652 | 0.534 | 1.87 | 7.33 | 8.8 |
| 0.9 | 0.624 | 0.524 | 1.91 | 7.47 | 9.0 |
| 1.0 | 0.585 | 0.536 | 1.87 | 7.30 | 9.0 |

Table 5. Dominant (slow) pole at Kp,min and the single-exponential settling-time estimate.

At Kp,min the closed loop has two real poles and the response is dominated by the slow one. Its decay rate σ ≈ 0.49–0.57 s⁻¹ is almost independent of T and equals the slow pole of the continuous loop at its own lower limit (0.484 s⁻¹): the lower edge is a property of the plant’s slow mode, not of the sampling. The single-exponential estimate ln 50/σ ≈ 7–8 s underestimates the simulated 8.4–9 s because the fast pole’s residue and the 2 % band add roughly one more time constant. The simulated ts is a multiple of T, so the lower edge moves in jumps — at T = 0.7 s the response settles at t = 8.4 s for Kp ≥ 0.521 but at 9.1 s just below it, which is why your sweep shows the non-monotone sequence 0.561, 0.561, 0.551, 0.531, 0.511, 0.531, 0.501 and why the exact edge is also irregular (Figure 4).

# 11. Empirical Kp(T) Relationships

Both boundaries fall with T roughly like a delayed analog design. A two-parameter law

was fitted by least squares to the exact edges. Upper edge: Kp0 = 1.5382, c = 0.9531, R² = 0.99987. Kp0 equals the analog limit 1.536 to 0.2 %, and c equals a/2 = 0.949 to 0.5 %: the zero-order hold acts as a half-sample delay, Kp,max(T) ≈ Kp,max,analog/(1 + aT/2), and this parameter-free prediction already gives R² = 0.99985. Lower edge: Kp0 = 0.5850 (analog 0.582), c = 0.2310, R² = 0.954; the delay law does not apply (R² < 0) because this edge is fixed by the slow pole (§10), not by phase lag, and the fit is only descriptive (scatter ±0.02 from the T-quantisation). Extrapolating the two fits would suggest the window closes near T ≈ 4.7 s; the direct computation of §13 shows it actually closes at T = 3 s (the settling-time limit of 9 s then equals three samples), so the fits should not be used beyond the tested range. The width follows ΔKp ≈ 1.538/(1+0.953T) − 0.585/(1+0.231T) (from 0.83 to 0.32).

# 12. Waterfall Analysis at T = 1 s

Seventy closed-loop step responses were generated for Kp from 0.1·Kp,min to 1.35·Kp,stab (csv_user_plant/waterfall_*.csv). Figure 6 (left) stacks them; the right panel plots the resulting overshoot and settling time against Kp.

Figure 6. Left: waterfall of step responses at T = 1 s (clipped at y = 2.2). Right: overshoot and settling time against Kp; shaded: admissible window; dashed: stability limit.

The picture shows three regimes. Below Kp,min the response is overdamped and slow (ts > 9 s). Inside the window ts is not monotone: it falls from 9 s at 0.468 to a minimum of 5 s near Kp ≈ 0.63–0.68 (overshoot ≈ 1.4–1.6 %), then rises again to 7 s at Kp,max as the oscillation starts to ring across the 2 % band. Above Kp,max overshoot grows steeply and ts becomes irregular (the staircase in the right panel is the 2 % band being re-crossed by successive peaks) until the loop reaches sustained oscillation at Kp,stab = 4.284. The decay rate σ and damped frequency of the ringing agree with the closed-form pole location (Table 6), confirming the z-plane picture; the gap near Kp,stab is the coarse sample grid (about four samples per cycle).

| Gain | Kp | σ theory (1/s) | σ from simulation | ωd theory (rad/s) | ωd from simulation |
| --- | --- | --- | --- | --- | --- |
| Kp,max | 0.785 | 0.593 | 0.601 | 0.622 | 0.634 |
| 2.0 | 2.000 | 0.302 | 0.312 | 1.219 | 1.273 |
| 3.0 | 3.000 | 0.147 | 0.149 | 1.461 | 1.605 |
| 0.95 Kp,stab | 4.070 | 0.022 | 0.047 | 1.649 | 1.723 |

Table 6. Pole-based prediction against measured ringing in the simulated step responses (T = 1 s).

# 13. Extended Heat Maps: the Complete (T, Kp) Analysis

The earlier maps covered only the tested range. Here every quantity of the analysis is evaluated over a much wider plane — T from 0.05 s to 6 s and Kp from 0.05 to 100, covering the overdamped region, the whole feasible window, the oscillatory region and the instability boundary — on a 130 × 190 grid, each point from a full discrete simulation (csv_user_plant/combo_heatmap_grid.csv). Figures 7 and 8 show eight quantities on identical axes; in every panel the white contour is OS = 5 %, the cyan contour is ts = 9 s, the dashed line is the stability limit Kp,stab(T) and the dotted verticals bound the tested range T = 0.1–1.0 s.

Figure 7. (a) overshoot, (b) 2 % settling time, (c) effective damping ζeff and (d) decay rate σ of the dominant pole over the (T, Kp) plane (log axes). Unstable points are left blank.

Figure 8. (e) overshoot sensitivity dOS/d ln Kp, (f) gain margin, (g) phase margin and (h) samples per oscillation 2π/(ωdT). The oblique stripes in (e) and the ragged ts contour at high gain come from the sampled oscillation peaks changing sample.

Reading the maps. (a, c) Overshoot is zero while the poles are real (below breakaway, ζeff = 1) and rises along bands of falling ζeff; the 5 % contour is the ζeff = 0.69 line, and both fall with T. (b, d) Settling time is shortest in a thin strip just above breakaway where σ is largest (≈ 0.7–0.9 s⁻¹) and grows on both sides: below it because the slow pole barely leaves z = 1 (σ → 0 as Kp → 0), above it because the complex pair approaches the unit circle. (e) The overshoot sensitivity is essentially zero up to breakaway and then jumps to 16–25 percentage points per unit ln Kp at the 5 % edge — the edge sits on this steep slope. (f, g) Margins are monotone in Kp: at the 5 % contour the gain margin is 15–23 dB and the phase margin ≈ 62° at every T. (h) At the 5 % contour the ringing spans about 68 samples per cycle at T = 0.1 s but only 10 at T = 1 s, and about 4 near the stability limit, which is the clearest picture of why coarse sampling erodes the design.

Figure 9. Combined map. Colour: regime (grey unstable; light blue stable but ts > 9 s; green feasible; orange OS > 5 % only; red-orange both violated). Lines: OS = 5 %, ts = 9 s, PM = 60°, GM = 10 and 20 dB, σ = 0.5 s⁻¹, breakaway and stability limit; markers: the exact edges of Table 3. The red vertical marks the closing of the window at T = 3 s.

| T (s) | Kp,min | Kp,max | window width | breakaway | Kp,stab | zero |
| --- | --- | --- | --- | --- | --- | --- |
| 1.0 | 0.469 | 0.785 | 0.316 | 0.488 | 4.284 | -0.620 |
| 1.2 | 0.466 | 0.729 | 0.264 | 0.452 | 3.886 | -0.542 |
| 1.5 | 0.422 | 0.633 | 0.211 | 0.406 | 3.514 | -0.452 |
| 2.0 | 0.405 | 0.467 | 0.062 | 0.346 | 3.195 | -0.345 |
| 2.5 | 0.385 | 0.405 | 0.021 | 0.301 | 3.052 | -0.274 |
| 2.8 | 0.349 | 0.381 | 0.033 | 0.279 | 3.007 | -0.242 |
| 2.9 | 0.338 | 0.375 | 0.037 | 0.272 | 2.996 | -0.233 |
| 3.0 | 0.328 | 0.367 | 0.039 | 0.266 | 2.986 | -0.224 |
| 3.5 | — | — | empty | 0.239 | 2.956 | -0.188 |
| 4.0 | — | — | empty | 0.217 | 2.943 | -0.162 |
| 6.0 | — | — | empty | 0.160 | 2.933 | -0.103 |

Table 7. The gain window beyond the tested range (csv_user_plant/extended_window_vs_T.csv; fine scan, step 0.0005).

What the combined map adds. (i) The feasible band is one connected region over the whole tested range and keeps narrowing beyond it — 0.316 at T = 1 s, 0.264 at 1.2 s, 0.211 at 1.5 s, 0.062 at 2 s, 0.021 at 2.5 s (Table 7). (ii) It closes completely at T = 3 s: since ts can only be a multiple of T, the 9 s limit corresponds to three samples at T = 3 s, a narrow window (0.328–0.367) remains, and for T > 3 s fewer than three samples are allowed and no gain satisfies both specifications, even though the loop is still stable up to Kp ≈ 3. (iii) Above the green band the overshoot violation is the only failure over a wide gain range (orange), and the gap to instability (grey) is a factor 5–15 at the tested T; below breakaway the failure is always settling time (light blue), never overshoot. (iv) The contour of σ = 0.5 s⁻¹ runs along the lower edge of the window for T ≲ 1 s, confirming that the lower edge is the slow-pole condition of §10, while the PM = 60° line lies just above the 5 % contour (PM ≈ 62° at its edge) and the GM = 10 dB line lies 1.7–4.6× higher in gain, so the overshoot edge is also the edge of a comfortable phase margin. (v) The non-minimum-phase zero (T < 0.453 s) leaves no visible mark on the maps: at fine sampling the design is governed by the same two edges.

# 14. Frequency-Domain Checks

| T (s) | GM at Kp,max (dB) | PM at Kp,max (°) | GM at Kp,min (dB) | PM at Kp,min (°) | ωb (rad/s) | π/T (rad/s) | (π/T)/ωb |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 | 23.2 | 63.7 | 31.1 | 78.2 | 1.302 | 31.42 | 24.1 |
| 0.2 | 20.1 | 63.3 | 27.4 | 77.5 | 1.239 | 15.71 | 12.7 |
| 0.3 | 18.2 | 63.0 | 25.1 | 76.8 | 1.184 | 10.47 | 8.8 |
| 0.4 | 17.1 | 62.8 | 23.3 | 75.9 | 1.124 | 7.85 | 7.0 |
| 0.5 | 16.2 | 62.6 | 22.2 | 75.5 | 1.073 | 6.28 | 5.9 |
| 0.6 | 15.7 | 62.5 | 21.3 | 75.0 | 1.023 | 5.24 | 5.1 |
| 0.7 | 15.2 | 62.3 | 20.2 | 73.8 | 0.984 | 4.49 | 4.6 |
| 0.8 | 15.0 | 62.4 | 19.9 | 73.7 | 0.935 | 3.93 | 4.2 |
| 0.9 | 14.8 | 62.2 | 19.6 | 73.4 | 0.902 | 3.49 | 3.9 |
| 1.0 | 14.7 | 62.3 | 19.2 | 73.0 | 0.856 | 3.14 | 3.7 |

Table 8. Gain/phase margins at both edges and closed-loop −3 dB bandwidth at Kp,max (csv_user_plant/B1_margins.csv, C1_bandwidth.csv).

Figure 10. Gain margin and phase margin at both edges against T, and the Nyquist plot at T = 1 s for Kp,max, a mid gain and Kp,stab (the last passes through −1; no encirclements for the first two).

The phase margin at Kp,max is almost independent of T (62–64°, the 5 % overshoot signature) while at Kp,min it is 73–78°; the gain margin falls from 23 dB to 14.7 dB (T = 0.1 → 1 s) at Kp,max and equals 20 log10(Kp,stab/Kp) by construction. At T = 1 s the Nyquist curve for Kp,max stays 0.69 from −1 with no encirclement, while the Kp,stab curve touches −1. The closed-loop bandwidth falls from 1.30 to 0.86 rad/s, whereas the Nyquist frequency π/T falls from 31 to 3.1 rad/s: the ratio drops from 24 to 3.7, so at T = 1 s the loop is sampled only about 3.7× faster than its bandwidth — far short of the usual 10–20×, which is why the digital limits differ so much from the analog ones at this T.

# 15. Sampling Physics: Zero, Sensitivity, Intersample

Zero migration. The single finite zero of Gd(z) is −b0/b1. For your plant it is −4.13 at T = 0.1 s, −1.69 at 0.2 s, crosses z = −1 at Tc = 0.453 s and ends at −0.62 at T = 1 s (Figure 11, right): for T < Tc the discrete plant is non-minimum-phase, reflecting the continuous zero at s = +31.18. It does not limit the design (Kp,stab ≫ Kp,max) but it is why the loci of Figure 2 bend towards a zero outside the unit circle at fine sampling and why the exact upper edge differs slightly from a zero-free second-order prediction.

Figure 11. Closed-loop bandwidth against the Nyquist frequency, bandwidth margin, and migration of the discrete zero with T.

Pole sensitivity. Implicit differentiation of the characteristic equation gives dz/dKp = −(b1z + b0)/[2z + Kpb1 − (1+p)]. At T = 1 s the dominant pole moves 0.555 per unit gain at Kp,max and 0.180 at Kp,stab (ratio 3.1): the pole is most sensitive to gain near the design edge and less so near instability, where it simply slides along the unit circle.

Intersample behaviour. stepinfo sees only the samples. Simulating the continuous plant under the held control with 60 sub-steps per sample (it reproduces the c2d response at the samples to 1e-14) gives, at Kp,max, a true continuous-time peak of 5.001–5.251 % against exactly 5.000 % at the samples — an excess of at most 0.25 percentage points (T = 0.9 s). The non-minimum-phase zero produces an initial undershoot of only 0.04–0.08 %. The sampled evaluation is therefore a sound basis for the specification.

Figure 12. Left: continuous output and sampled output at T = 1 s, Kp,max. Right: continuous-time vs sampled overshoot against T.

# 16. Robustness and Lead Compensation

Robustness. With the gain placed mid-window (Kp = 0.626 at T = 1 s, 0.987 at T = 0.1 s) the plant was perturbed by ±10 % in gain (c0, c1) and in the pole a, in all eight corners plus the single-parameter cases, and re-discretised. Every case still meets both specifications; the worst case is a −10 % pole with +10 % gain: overshoot 3.36 %, ts 8 s at T = 1 s (2.66 %, 5.4 s at T = 0.1 s). Choosing Kp at Kp,max instead would violate the 5 % limit for any positive gain error (+10 % → 6.7–7.7 %), so the mid-window gain is the sensible design point.

Lead compensator (T = 1 s). Because the plant pole p = 0.15 is known, a lead C(z) = K(z − 0.15)/(z − pc) cancels it, leaving L = K b1(z + 0.62)/[(z − 1)(z − pc)]. The best proportional design reaches ts = 5 s (Kp ≈ 0.635–0.678, overshoot ≈ 1.4 %). Moving the compensator pole to the left shortens ts at higher gain (Table 9): pc = 0.05 gives 4 s at K = 0.84 (OS 1.4 %), pc = −0.10 gives 3 s and pc ≤ −0.30 gives 2 s, at the price of a loop gain 2–3× larger (more control effort and noise gain, and a smaller stability reserve). The first design that meets 4 s with ≤ 5 % overshoot (pc = 0.05) is shown in Figure 13 (right).

for pl = 0.15:-0.05:-0.60 % lead pole; zero fixed at 0.15 (cancels the plant pole) for K = 0.05:0.01:3 % loop L = K*b1(z+0.62)/((z-1)(z-pl)) den = [1, K*b1-(1+pl), pl+K*b0]; if any(abs(roots(den))>=1), continue, end y = filter([0 K*b1 K*b0],den,ones(61,1)); OS = max(0,(max(y)-1)*100); k = find(abs(y-1)>0.02,1,'last'); ts = k; % T = 1 s: samples are 1 s apart if OS <= 5 && ts < best_ts, best = [K OS ts]; best_ts = ts; end end end

| pc (lead pole) | K | OS (%) | ts (s) |
| --- | --- | --- | --- |
| +0.15 | 0.64 | 1.19 | 5 |
| +0.05 | 0.84 | 1.38 | 4 |
| -0.10 | 1.18 | 1.71 | 3 |
| -0.20 | 1.33 | 0.22 | 3 |
| -0.30 | 1.74 | 1.74 | 2 |
| -0.40 | 1.90 | 0.00 | 2 |

Table 9. Lead designs at T = 1 s (zero at 0.15; K chosen for minimum ts with OS ≤ 5 %; full list in lead_search.csv). pc = 0.15 is the plain proportional controller.

Figure 13. Left: overshoot and settling time for the eight ±10 % plant-error corners at T = 1 s (mid-window gain). Right: best proportional response against the lead design.

# 17. MATLAB Implementation of the Added Analysis

The complete code is in the accompanying package (matlab_user_method, 15 files; run verify_user_method first, then main_experiment7_user). It was cross-checked against an independent Python implementation but could not be run in MATLAB where it was prepared. Plotting, CSV export and bookkeeping loops are not reproduced; the three routines that are new for your plant are.

(a) Plant chain. Closed-form equivalent of your d2c / c2d steps (the main script asserts agreement with c2d at every T):

[a,c0,c1,N1,N0] = plant_from_sheet(0.32, 0.32*0.62, 0.15, 1); % Gs = (N1 s+N0)/(s^2+a s) [b1,b0,p] = zoh_from_Gs(T,a,c0,c1); % p=exp(-aT); g=c1(1-p)/a; b1=c0T+g; b0=-c0Tp-g

(b) Fast step metrics (same definitions as stepinfo, base MATLAB only; evaluate_design gives the stepinfo version for cross-checking):

den = [1, Kp*b1-(1+p), p+Kp*b0]; if any(abs(roots(den))>=1), OS=Inf; ts=NaN; return, end % unstable y = filter([0 Kp*b1 Kp*b0], den, ones(n,1)); t = (0:n-1)'*T; OS = max(0,(max(y)-1)*100); k = find(abs(y-1)>0.02,1,'last'); ts = t(k+1); % last sample outside 2% band

(c) Exact gain window (fine scan, then bisection on each edge):

ok_fun = @(K) test_ok(b1,b0,p,K,T,OS_spec,ts_spec); % OS<=5 && ts<=9 Ks = 0.2:0.001:2.4; idx = find(arrayfun(ok_fun,Ks)); Kmin = bisect_edge(ok_fun, Ks(idx(1)-1), Ks(idx(1))); % bad -> good Kmax = bisect_edge(ok_fun, Ks(idx(end)+1), Ks(idx(end)));

The root-locus helpers used by this analysis (breakaway_gain, zeta_error, zeta_crossing_gain, marginal_stability_gain, fit_inverse_law) are unchanged and are described, with their code, in §C2.4–C2.8; the lead search of §16 is in section 10 of the main script.

# 18. Data and Code Availability

All numbers in Part II come from the CSV files in csv_user_plant/: table1_digital_design (all gain edges, boundaries, your values alongside), table2_ts_at_Kpmin, A3_interval_width, B1_margins, C1_bandwidth, C2_zero_migration, D2_robustness, E_intersample_vs_T, lead_search, waterfall_stepresponse_T1s, waterfall_OS_ts_vs_Kp, waterfall_theory_validation and combo_heatmap_grid and extended_window_vs_T. The Python cross-check reproduces your sweep: every one of your twenty Kp values lies within 0.016 of the exact edge.

— Part III: Companion Study with the Analytic Model K/[s(s+a)] —

This part reproduces selected material of the earlier report, which modelled the plant analytically as K/[s(s+a)] (a = 1.8971, K = 1.0999 fitted from the pole and leading coefficient of the sheet) and used the sheet’s own coefficients at T = 1 s. It differs from Parts I–II only in the plant used for T ≠ 1 s. Its tables and figures are labelled with a C prefix (Table C0, Figure C1, §C4.1) so they cannot be confused with those of Parts I–II; references to “§14–§16” and “§19–§21” point back into Parts II and IV.

# C1. Theory of the Analytic Model

Recovering the plant. The pole at z = 1 is a free integrator, so the continuous plant is modelled as K/[s(s+a)]. The finite pole obeys z = e−aT, so the sheet’s pole 0.15 at T = 1 s gives a = −ln 0.15 = 1.8971 rad/s, and matching the leading ZOH coefficient b1 = 0.32 gives K = 1.0999. With proportional gain Kp the continuous characteristic equation is s² + as + KpK = 0, which gives the analog gain–damping relation and, for ZOH discretisation at any T, the closed-form coefficients below.

The ZOH acts roughly like a T/2 pure delay, which should erode the overshoot-limited gain as Kp0/(1 + aT/2); the fit of §C4.3 and §11 tests this heuristic. The analytic model has no non-minimum-phase zero: its ZOH zero tends to −1 as T → 0, whereas the d2c → c2d chain of Parts I–II keeps the sheet’s zero at s = +31.18 (§15).

# C2. MATLAB Implementation of the Analytic Model

## C2.1 Recovering G(s) and verifying by discretisation (Objective 1)

Objective: Recover a and K from the supplied G(z) at T = 1 s, build the continuous plant, and check it against the supplied G(z) when re-discretised.

% --- Recover the analog plant from the sampled model at T = 1 s --- T1 = 1; pz = 0.15; b1_target = 0.32; zero_given = 0.62; % lab-sheet values a = -log(pz)/T1; % z = exp(-aT) -> a = 1.8971 rad/s p1 = exp(-a*T1); K = b1_target*a^2/(a*T1 - 1 + p1); % match leading ZOH coefficient -> K = 1.0999 Gs = tf(K,[1 a 0]); % recovered plant K/[s(s+a)] Gz_check = c2d(Gs, T1, 'zoh'); % ZOH model: 0.32 z + 0.1728 over (z-1)(z-0.15) nz = Gz_check.Numerator{1}; % [0 b1 b0] fprintf('model zero %.4f vs sheet zero %.4f\n', -nz(3)/nz(2), -zero_given);

Running this gives a = 1.8971 rad/s and K = 1.0999. The re-discretised plant reproduces the sheet’s pole (0.15) and leading coefficient (0.32) by construction, but gives b0 = 0.1728, i.e. a zero at −0.5400 rather than the sheet’s −0.62 (b0 = 0.1984). This 14.8% mismatch is analysed in §C3.1.

## C2.2 Analog proportional-gain range (Objective 2)

Objective: Convert the 5% overshoot spec to ζmin, then use Kp(ζ) = a²/(4Kζ²) for the analog Kp interval and the (constant) ideal settling time.

OS_spec = 5; % percent overshoot limit zeta_min = -log(OS_spec/100)/sqrt(pi^2+log(OS_spec/100)^2); % 0.6901 Kp_max_analog = a^2/(4*K*zeta_min^2); % = 1.7177 Kp_min_analog = a^2/(4*K*1^2); % = 0.8181 ts_analog = 8/a; % 4/(zeta*wn), zeta*wn = a/2 -> 4.2169 s

## C2.3 ZOH re-discretisation for a general T (Objective 3, part 1)

Objective: Provide the plant’s ZOH numerator coefficients at any sampling time so the same design procedure runs for every T of the sweep.

function [b1,b0,p] = zoh_coeffs(T,a,K) % ZOH-equivalent coefficients of K/[s(s+a)] (same as c2d(...,'zoh'), closed form) p = exp(-a*T); b1 = (K/a^2).*(a*T - 1 + p); b0 = (K/a^2).*(1 - p - a*p.*T); end

## C2.4 Locating the ζ = 1 boundary exactly (Objective 3, part 2)

Objective: Find the breakaway gain, where the two real closed-loop poles meet and turn complex, without plotting the locus.

The locus leaves the real axis where the discriminant of z² + [Kpb1−(1+p)]z + [p+Kpb0] vanishes:

function [Kp_min,Kp_breakin] = breakaway_gain(b1,b0,p) A = b1^2; B = -(2*b1*(1+p) + 4*b0); C = (1-p)^2; r = sort(roots([A B C])); % two real roots: breakaway, break-in Kp_min = r(1); Kp_breakin = r(2); % smaller one is the zeta = 1 boundary end

## C2.5 Locating the ζ = 0.69 boundary (Objective 3, part 3)

Objective: Find the gain at which the complex-pole branch crosses the 5%-overshoot contour, using the T-independent ζeff of §C1.3.

function Kp_max = zeta_crossing_gain(b1,b0,p,target_zeta,Kp_lo,Kp_hi) Kp_max = fzero(@(Kp) zeta_error(Kp,b1,b0,p,target_zeta), [Kp_lo Kp_hi]); end function e = zeta_error(Kp,b1,b0,p,target) r = roots([1, Kp*b1-(1+p), p+Kp*b0]); zc = r(imag(r) > 0); % upper complex pole e = -log(abs(zc))/abs(log(zc)) - target; % zeta_eff(z) - target end

## C2.6 Performance evaluation (Objective 4)

Objective: Given a design point (Kp, T), build the closed-loop discrete transfer function and read off the true overshoot and settling time.

function [OS,ts] = evaluate_design(b1,b0,p,Kp,T) Gz = tf([b1 b0],[1 -(1+p) p], T); % open-loop ZOH plant Gcl = feedback(Kp*Gz, 1); % unity feedback info = stepinfo(Gcl,'SettlingTimeThreshold',0.02); OS = info.Overshoot; ts = info.SettlingTime; end

## C2.7 Marginal-stability gain (further analysis, §C4.1)

Objective: Compute the exact upper stability limit on Kp from the closed-form result of §C1.4.

function Kp_stab = marginal_stability_gain(b0,p) % On the complex branch ¦z¦^2 = C = p + Kp*b0; instability begins when it reaches 1 Kp_stab = (1 - p)/b0; end

## C2.8 Empirical Kp(T) relationship (Objective 5, further analysis)

Objective: Fit a closed-form approximation to how Kp,min and Kp,max fall with sampling time, and report the goodness of fit.

function [Kp0,c,R2] = fit_inverse_law(Tvec,Kpvec) % Fit Kp(T) ~= Kp0/(1+c*T) by nonlinear least squares (fminsearch: base MATLAB) model = @(par,T) par(1)./(1+par(2).*T); sse = @(par) sum((Kpvec(:) - model(par,Tvec(:))).^2); par = fminsearch(sse,[Kpvec(1),1],optimset('TolX',1e-12,'TolFun',1e-14,'MaxIter',5000)); Kp0 = par(1); c = par(2); R2 = 1 - sum((Kpvec(:)-model(par,Tvec(:))).^2)/sum((Kpvec(:)-mean(Kpvec)).^2); end

# C3. Sheet versus Analytic Model: Consistency and Comparison

## C3.1 Recovered plant, analog design, and the consistency of the supplied G(z)

§C2.1 gives a = 1.8971 rad/s and K = 1.0999, so G(s) = 1.0999/[s(s+1.8971)]. §C2.2 gives ζmin = 0.6901, an analog gain range 0.8181 ≤ Kp ≤ 1.7177 and a constant ideal settling time 8/a = 4.2169 s across that range (ζωn = a/2 is fixed by construction).

Consistency of the supplied G(z) (new finding). For a plant K/[s(s+a)] the ZOH numerator coefficients b1 and b0 and the pole p are tied together by a single pair (a, K). The sheet gives three numbers (p = 0.15, b1 = 0.32, zero −0.62), so one of them is redundant, and it is inconsistent: p and b1 fix a and K, which then force the zero to

a 14.8% difference. The effect on the design is not negligible (Table C0). The sheet’s own coefficients are used for every T = 1 s analysis (in the later sections) because that is the plant the lab specifies, while the T-sweep necessarily uses the recovered continuous model. A zero of 0.54 on the sheet would make the two descriptions coincide exactly, so it is worth re-checking the handwritten value; the MATLAB switch USE_GIVEN_AT_T1 (or editing zero_given) re-runs everything either way.

| T = 1 s | Zero z0 | Kp,min (ζ=1) | Kp,max (ζ=0.69) | Kp,stab | Kp,stab/Kp,max | OS / ts at Kp,max |
| --- | --- | --- | --- | --- | --- | --- |
| Sheet G(z) [0.32(z+0.62)] | −0.6200 | 0.4883 | 0.7845 | 4.284 | 5.46× | 5.00% / 7 s |
| ZOH model [0.32(z+0.540)] | −0.5400 | 0.5261 | 0.8524 | 4.919 | 5.77× | 4.98% / 7 s |
| Difference (sheet vs model) | +14.8% | -7.2% | -8.0% | -12.9% |  |  |

Table C0. The same design procedure applied to the sheet’s G(z) and to the exact ZOH model of the recovered plant, at T = 1 s. Both reach ts = 9.0 s at Kp,min and 7 s at Kp,max, so the conclusions are unchanged; the gains shift by 7–13%.

Figure C1. Left: the sheet’s zero (−0.62) and the zero of the exact ZOH model (−0.540). Right: step responses, each at its own Kp,max: the two are almost indistinguishable at the sample instants.

| T (s) | Kp,max d2c→c2d (Part I) | Kp,max analytic (§C3) | Δ | Breakaway d2c→c2d | Breakaway analytic | Δ | zero d2c→c2d | zero analytic |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 | 1.4043 | 1.5613 | +11.2 % | 0.7205 | 0.7803 | +8.3 % | -4.13 | -0.94 |
| 0.2 | 1.2913 | 1.4299 | +10.7 % | 0.6881 | 0.7447 | +8.2 % | -1.69 | -0.88 |
| 0.3 | 1.1984 | 1.3182 | +10.0 % | 0.6575 | 0.7111 | +8.2 % | -1.27 | -0.83 |
| 0.4 | 1.1121 | 1.2224 | +9.9 % | 0.6286 | 0.6794 | +8.1 % | -1.07 | -0.78 |
| 0.5 | 1.0411 | 1.1395 | +9.4 % | 0.6014 | 0.6497 | +8.0 % | -0.95 | -0.73 |
| 0.6 | 0.9766 | 1.0671 | +9.3 % | 0.5759 | 0.6217 | +8.0 % | -0.85 | -0.69 |
| 0.7 | 0.9259 | 1.0035 | +8.4 % | 0.5519 | 0.5955 | +7.9 % | -0.78 | -0.65 |
| 0.8 | 0.8709 | 0.9473 | +8.8 % | 0.5293 | 0.5709 | +7.8 % | -0.72 | -0.61 |
| 0.9 | 0.8319 | 0.8972 | +7.8 % | 0.5082 | 0.5478 | +7.8 % | -0.67 | -0.57 |
| 1.0 | 0.7846 | 0.8524 | +8.6 % | 0.4883 | 0.5261 | +7.7 % | -0.62 | -0.54 |

Table C1. The two plant descriptions side by side. They coincide in pole and leading coefficient at T = 1 s but differ in the zero (−0.62 vs −0.540), and for T < 1 s the d2c→c2d chain keeps the sheet’s non-minimum-phase zero while the analytic model has none; the Kp,max values differ by 8–11 %.

Which description to use. The chain used in your report reproduces the lab sheet exactly at T = 1 s (zero −0.62), so its T = 1 s numbers (0.468–0.785) are the ones to quote; the analytic model assumes a plant form the sheet does not obey exactly, and gives 0.488–0.852 at T = 1 s if its own ZOH zero is used (Table C0). The two models agree on every qualitative conclusion — gain windows that shrink with T, a half-sample-delay law for Kp,max, stability limits several times above the design gain — and on the closed-form tools; they differ by a roughly constant 8–11 % in Kp,max. The analytic model is useful because K/[s(s+a)] has closed-form analog formulas (§C2.2) and an unconditional-stability argument (§C4.1).

# C4. Further Analysis with the Analytic Model

## C4.1 How close is the design to instability?

The open-loop model has one more pole than zero, so the root locus has a single asymptote and, as Kp grows, one closed-loop pole heads to −∞ while the other approaches the open-loop zero. In between, the complex pair crosses the unit circle at a single exact gain Kp,stab = (1−p)/b0 (§C2.7). For the sheet’s plant at T = 1 s this is Kp = 4.284: 5.46× the gain that already gives 5% overshoot (0.7845). On the recovered model the margin ratio grows from 5.77× at T = 1 s to 22.8× at T = 0.1 s (csv_new_plant/table1_digital_design.csv).

This is an important, slightly counter-intuitive digital-control fact. The analog plant K/[s(s+a)] under proportional feedback is unconditionally stable for every Kp > 0 (s² + as + KpK has positive coefficients). Sampling destroys this: ZOH sampling of a type-1 plant introduces a finite zero, giving a real asymptote and a finite stability limit, so gains harmless in continuous time can destabilise the sampled loop. The 5% overshoot limit lies well inside the stability boundary over the whole tested range, but a designer relying on continuous-time intuition would be caught out at higher gains.

## C4.2 The critical-damping settling-time correction

The usual ts ≈ 4/(ζωn) comes from the 1/√(1−ζ²) envelope of an underdamped second-order response and is only valid away from ζ = 1. At the Kp,min boundary the closed-loop poles are real and repeated, the natural mode is n·rⁿ, and the 2%-settling constant becomes the root of

| T (s) | ts naive (4/σ) | ts corrected (5.83/σ) | ts full simulation |
| --- | --- | --- | --- |
| 0.1 | 4.321 | 6.302 | 6.4 |
| 0.4 | 4.682 | 6.828 | 7.2 |
| 0.7 | 5.117 | 7.463 | 7.7 |
| 1.0 | 5.621 | 8.198 | 9.0 |

Table C2. Settling time at the Kp,min (ζ = 1) boundary (all ten T in csv_new_plant/table2_ts_at_Kpmin.csv). The corrected formula tracks the simulation (to within the one-sample quantisation); at T = 1 s the true settling time (9.0 s) sits exactly on the 9 s limit, not comfortably inside it as the naive 5.62 s suggests.

Practical takeaway: near ζ = 1 use 5.83/σ (or a full simulation). The gap is small at fine sampling but is the difference between comfortably meeting the specification and sitting exactly on its boundary once T approaches 1 s. The same 9.0 s result holds for the sheet’s own G(z) (Table C0).

## C4.3 Empirical Kp(T) relationship

Fitting Kp(T) ≈ Kp0/(1+cT) by nonlinear least squares (§C2.8) to the two design boundaries gives

| Quantity | Fitted model | R² |
| --- | --- | --- |
| Kp,max(T) (ζ = 0.69) | 1.7212 / (1 + 1.0205·T) | 1.00000 |
| Kp,min(T) (ζ = 1, breakaway) | 0.8288 / (1 + 0.5631·T) | 0.99895 |
| ts,max(T) (linear, simulated) | 2.764·T + 4.400 | 0.9611 |
| ts,min(T) (linear, simulated) | 2.497·T + 6.147 | 0.9428 |

Table C3. Empirical curve fits. Both Kp fits are excellent, and their zero-sampling intercepts Kp0 (1.721 and 0.829) land almost exactly on the analog values (1.718 and 0.818) of §C3.1, so the digital design converges to the analog design as T → 0.

# C5. Lab-Manual Graphical Workflow

The lab manual asks for the graphical workflow (rlocusplot with zgrid, datatips, Control System Designer) in addition to the analytic route. The steps below reproduce the T = 1 s and T = 0.5 s designs graphically and give the targets a careful user should read off.

## C5.1 Graphical zgrid workflow, T = 1 s

Objective: Use rlocusplot with a zgrid(0.69,[]) overlay, read the gain at the ζ = 0.69 crossing via a datatip, and cross-check against the analytic Kp,min = 0.4883 / Kp,max = 0.7845.

T = 1; Gz1 = tf(0.32*[1 0.62],conv([1 -1],[1 -0.15]),T); % sheet G(z) h = rlocusplot(Gz1); hold on; zgrid(0.69,[]); % zeta = 0.69 spiral + unit circle opts = getoptions(h); opts.Title.String = 'Root locus, \zeta = 0.69 contour, T = 1 s'; setoptions(h,opts); % Click the branch where it meets the dashed zeta=0.69 curve: datatip shows Gain, Damping, Overshoot. [rl,kl] = rlocus(Gz1); % programmatic cross-check of the datatip zeta_vec = nan(1,numel(kl)); for q = 1:numel(kl) zc = rl(imag(rl(:,q)) > 1e-9, q); if ~isempty(zc), zeta_vec(q) = effective_zeta(zc(1)); end end [~,iq] = min(abs(zeta_vec - 0.6901)); fprintf('Kp,max graphical %.4f ¦ analytic %.4f\n', kl(iq), Kp1_max);

The locus starts at z = 1 and z = 0.15, breaks away at z ≈ 0.497 and crosses the ζ = 0.69 spiral near z ≈ 0.45 + 0.32j (Figure C2). The datatip there should read about Gain 0.78, Damping 0.690, Overshoot 5.0%. A mouse reading is limited to roughly ±1%, and rlocus’s gain vector is not refined at a contour crossing the way the fzero routine of §C2.5 is, so a small discrepancy is expected; the graphical workflow is a sanity check on the analytic pipeline, not the source of the final numbers. The script prints the programmatic difference when run.

| Method | Kp,max (ζ = 0.69) | Kp,min (ζ = 1, breakaway) |
| --- | --- | --- |
| Graphical (datatip) | ≈ 0.78 (± 1%, cursor-limited) | ≈ 0.49 (± 1%) |
| Analytic (§C2.4–4.5) | 0.7845 | 0.4883 |

## C5.2 Control System Designer walkthrough, T = 0.5 s

Objective: Step-by-step controlSystemDesigner('rlocus',Gz) procedure at T = 0.5 s: import the plant, enable the damping grid, drag the pole onto the ζ = 0.69 contour and read the gain.

[b1c,b0c,pc] = zoh_coeffs(0.5,a,K); % recovered model at T = 0.5 s Gz_csd = tf([b1c b0c],[1 -(1+pc) pc], 0.5); controlSystemDesigner('rlocus', Gz_csd)

Launch. The Root Locus Editor shows poles z = 1 and z = 0.3873 and the zero at z ≈ −0.7304. [screenshot: window on open]

Enable the damping grid (right-click → Grid), then add a design requirement (right-click → Design Requirements → New → Damping Ratio = 0.69); a shaded exclusion wedge appears. [screenshot]

Drag the closed-loop pole square along the locus until it sits on the ζ = 0.69 boundary; the Compensator Editor updates live and should settle near Kp ≈ 1.14. [screenshot]

Repeat for ζ = 1: drag to the real-axis breakaway point; the readout should land near Kp ≈ 0.65. [screenshot]

These targets (Kp,min(0.5) = 0.6497, Kp,max(0.5) = 1.1395 from csv_new_plant/table1_digital_design.csv) let a mismatch of more than a percent or two be diagnosed as a mis-drag or a units error (Hz vs rad/s, or an un-discretised G(s)) instead of a real discrepancy.

— Part IV: Closing Sections —

# 19. Discussion

The results demonstrate the influence of sampling time on the design of a digital proportional controller. Although the same continuous plant underlies every case, its discrete representation changes with T through z = esT, so the discrete poles and zero — and with them the root-locus trajectory — change, and the gain must be chosen for the sampling time in use.

The two specifications bound the permissible gain: ts ≤ 9 s sets the lower bound and %OS ≤ 5 % the upper one. The added analysis makes this precise: the lower edge is a slow-pole effect (σ ≈ 0.5 s⁻¹ at every T, §10) that is reached while both poles are still real (below breakaway), hence 0 % overshoot there; the upper edge is where the complex branch crosses ζ = 0.6901, which the closed-form crossing reproduces to within 1 % (§9). An acceptable range exists for every T from 0.1 s to 1.0 s.

The maximum permissible gain decreases significantly with T: ◆ from 1.404 at T = 0.1 s to 0.785 at T = 1.0 s (your sweep: 1.401 → 0.781). The corresponding sentence of your report quotes 1.676134 and 0.921074, which do not match your own table or any of the exact values; the table values are used here. The decrease follows Kp,max(T) ≈ 1.538/(1 + 0.953T) with R² = 0.9999 — an analog limit divided by a half-sample ZOH delay (c ≈ a/2) — which also explains why larger sampling intervals leave a narrower window: the range shrinks by 62 % from T = 0.1 s to 1 s, mainly through the upper edge.

◆ Analog versus digital values. The analog limits are 0.582 ≤ Kp ≤ 1.536 (your quoted 0.678 / 1.845 do not satisfy the specifications for this plant, §4). The digital limits converge towards these as T → 0 (ζ-crossing gain 1.522 at T = 0.01 s), so the analog result should be compared with the T = 0.1 s case rather than the T = 1 s case, as your report notes; the apparently “extreme” difference is the sampling effect, not an error in the analog calculation.

Stability, robustness and limits of the conclusions. The true stability limit is 5.5×–14.5× above Kp,max (§9), the phase margin at Kp,max is ≈ 62°, the continuous-time overshoot exceeds the sampled one by at most 0.25 points (§15), and a mid-window gain survives ±10 % plant error at T = 1 s and T = 0.1 s (§16). Settling time is evaluated on the sample sequence and is quantised to multiples of T, which makes the lower edge irregular (T = 0.7 s). Results are for T = 0.1–1.0 s and the plant chain d2c → c2d; at T = 1 s the sheet G(z) is reproduced exactly, so no sheet-versus-model inconsistency arises. Within the window the fastest response (ts = 5 s) occurs mid-window near Kp ≈ 0.63–0.68, and a lead compensator cancelling the plant pole can reduce this to 4 s or less at higher gain (§16).

Two plant descriptions, one set of conclusions. Part III repeats the study with the analytic plant K/[s(s+a)]. It adds three results that carry over: the continuous plant is unconditionally stable for every Kp > 0, so the finite stability limit of the sampled loop is purely a sampling effect (§C4.1); near ζ = 1 the settling time must use the repeated-pole constant 5.83/σ rather than 4/σ (§C4.2); and Kp,max(T) follows Kp0/(1 + cT) with c ≈ a/2 in both descriptions, the lower edge in neither. The descriptions differ by 8–11 % in Kp,max (Table C1), because the analytic model lacks the sheet’s zero; the d2c → c2d chain of your report matches the sheet exactly at T = 1 s and is the one whose numbers should be quoted.

# 20. Observations

The damping ratio for 5 % peak overshoot is ζ = 0.6901.

The digital transfer function changes with every sampling time (Table 1); its zero moves from −4.13 to −0.62 and crosses the unit circle at Tc = 0.453 s.

The discrete root locus therefore changes with sampling time.

An acceptable proportional gain range exists for all T from 0.1 s to 1.0 s.

The minimum permissible gain is governed by the settling-time requirement and is reached with two real poles (below breakaway).

The maximum permissible gain is governed by the overshoot requirement and coincides (within 1 %) with the ζ = 0.6901 crossing.

The overshoot at the minimum gain is 0 % at every T; at the maximum gain it is 5 % (4.77–4.996 % in the coarse sweep).

The maximum gain falls from 1.404 to 0.785; the minimum gain from 0.569 to 0.468; the window narrows from 0.835 to 0.316.

Settling time at the maximum gain rises from 4.8 s to 7.0 s as T grows from 0.1 s to 1 s; inside the window ts is smallest (5 s) near Kp ≈ 0.63–0.68 at T = 1 s.

The stability limit is 5.5×–14.5× the maximum design gain; the phase margin at Kp,max is ≈ 62° at every T; the gain margin falls from 23 dB to 14.7 dB.

Kp,max(T) follows Kp0/(1 + cT) with Kp0 ≈ analog limit and c ≈ a/2 (R² = 0.9999); the lower edge does not follow this law.

A ±10 % plant error at mid-window gain keeps both specifications met; a gain at the overshoot edge would not survive a +10 % gain error.

In the analytic model the continuous loop is unconditionally stable, so every stability limit of the sampled loop is created by sampling; the 2 % settling time near ζ = 1 follows 5.83/σ, not 4/σ (§C4).

A lead compensator cancelling the plant pole (z = 0.15) reduces ts from 5 s (best proportional) to 4 s with ≤ 5 % overshoot at T = 1 s, and to 2–3 s at 2–3× the gain (§16).

The extended heat maps show one connected feasible band that keeps narrowing beyond T = 1 s and closes completely at T = 3 s, where 9 s equals three samples; above the band overshoot is the only failure, below it settling time (§13).

The graphical zgrid / Control System Designer workflow reproduces the analytic gains to the ±1 % limit of a cursor reading (§C5).

# 21. Conclusion

The proportional controller design was carried out with discrete root-locus analysis for sampling times from 0.1 s to 1.0 s. For every T the digital plant was obtained by zero-order-hold discretisation of the continuous equivalent of the given G(z), and the gain was varied to find the range meeting %OS ≤ 5 % and ts ≤ 9 s.

The permissible range changes from 0.571 ≤ Kp ≤ 1.401 at T = 0.1 s to 0.471 ≤ Kp ≤ 0.781 at T = 1.0 s in the lab sweep (exact limits 0.569–1.404 and 0.468–0.785). Increasing the sampling time therefore reduces the allowable gain range, mainly through the upper limit, which falls as Kp0/(1 + cT) with c ≈ a/2. The analysis added here shows that the design is well inside the stable region (5.5×–14.5× margin), robust to ±10 % plant error when the gain is chosen mid-window, and that a gain of about 0.63–0.68 at T = 1 s gives the fastest response (5 s) with about 1.5 % overshoot. The experiment demonstrates why sampling time must be considered when selecting a proportional gain for a digital root-locus design.

The extended heat maps show the complete picture at once: the window narrows with T and closes at T = 3 s, overshoot is the only failure above it and settling time the only failure below it. Three practical points follow: choose a gain inside the window (about 0.63–0.68 at T = 1 s) rather than at the overshoot edge, use 5.83/σ rather than 4/σ for settling time near ζ = 1, and consider a pole-cancelling lead compensator if a settling time below 5 s is needed. Where the analytic model and your d2c → c2d chain differ (8–11 % in Kp,max), your chain is the one that reproduces the lab sheet exactly.